# β-Amylase Purification and Activity Assay

## Process flow

| — | Process | In | Out | Key parameter |  | Week |
| --- | --- | --- | --- | --- | --- | --- |
| A | Crude extraction | 20 g tuber + 60 mL PBS | 60 mL Extract 1 | 1:3 w/v, 0–4 °C | 1 |  |
| B | Salt precipitation | 20 mL Extract 2 + 46.66667 mL sat. (NH₄)₂SO₄ | pellet → 1.0 mL Crude Enzyme 2 | 70 % saturation | 1 |  |
| C | Dialysis | 1.0 mL Crude Enzyme 2, 6–12 kDa tubing | desalted Crude Enzyme 2 | 6 h, 4 °C | 1–2 |  |
| D | Gel filtration | 1.0–1.6 mL clarified crude | 12 × 4.0 mL fractions | Vc = 80 mL, gravity | 2 |  |
| E | DNS activity assay | 0.5 mL diluted enzyme + 0.5 mL 1 % starch | A₅₄₀ → µmol → U/mL | 37 °C, 10 min | 3, 4 |  |
| F | Total protein, A₂₈₀ | 12 fractions + stages | A₂₈₀ elution profile | 280 nm | 3 |  |
| G | Electrophoresis | 10–20 µg per lane | bands at ~56 kDa subunit | SDS-PAGE 12 % | 3 |  |
| H | Protein Quantification | Enzyme fractions & extracts | Protein conc. (mg/mL) & Specific Activity (U/mg) | 280nm, 260nm or BSA | 3 |  |
| I | Enzyme Immobilization | Purified enzyme + Na-alginate | Ca-alginate beads | CaCl2, Na-alginate, 4 °C | 4 |  |

Temperature. Everything except the DNS boiling step stays at 0–4 °C.

## Materials

P1 = prep session, the day before Week 1.

### Full material list

Everything the whole protocol consumes. Quantities are totals — buy with a margin for one re-run.

#### Chemicals

| Reagent | Total | Used for |
| --- | --- | --- |
| NaCl | 32.02512 g | PBS 4 L |
| KCl | 0.80514 g | PBS 4 L |
| Na₂HPO₄ anhydrous | 5.6784 g | PBS 4 L |
| KH₂PO₄ | 0.979848 g (PBS) + 3.40225 g (assay buffer) | PBS, assay buffer |
| HCl, 1 M | ≈ 11.3362836 mL | PBS pH 7.4 |
| NaOH, 2 M | 20 mL; make 100 mL from 8 g NaOH | DNS reagent |
| (NH₄)₂SO₄ | 35.5 g (buy 100 g) | Saturated stock |
| Na₂HPO₄·12H₂O | 8.9535 g | Assay buffer 500 mL |
| CaCl₂ (optional) | 0.05549 g + ~2.22 g (for immobilization) | Assay buffer, 1 mM & Immobilization |
| Soluble starch | 2 g — 1 g per 100 mL, two batches | DNS assay, weeks 3 and 4 |
| D-maltose | 100.0 mg | Maltose standard 100 mL |
| 3,5-Dinitrosalicylic acid | 1 g | DNS reagent |
| Rochelle salt (NaK tartrate) | 30 g | DNS reagent |
| Distilled water | 5 L | all |
| Sodium alginate | 2-3 g | Enzyme immobilization |

#### Electrophoresis chemicals

Enough for two 1.0 mm, 10-well mini-gels.

| Reagent | Amount |
| --- | --- |
| Acrylamide/bis 30 % (29:1), resolving | ~8 mL (12 %) |
| Acrylamide/bis 30 % (29:1), stacking | ~2 mL (4 %) |
| Tris-HCl 1.5 M pH 8.8 | ~22 mL — 5 mL resolving gel, 16.7 mL running buffer |
| Tris-HCl 0.5 M pH 6.8 | ~6.25 mL — 1.25 mL stacking gel, 5 mL Laemmli buffer |
| SDS | ~1.1 g — 1.0 g running buffer, rest in the gels |
| Glycine | 14.4 g — running buffer |
| Glycerol | ~2 mL — Laemmli buffer |
| APS | ~0.1 g — buy extra, it must be fresh |
| TEMED | ~20 µL |
| Bromophenol blue | ~2 mg |
| β-Mercaptoethanol | ~0.4 mL — add fresh, in the hood |
| β-Mercaptoethanol | ~0.4 mL — add fresh, in the hood |
| Coomassie Brilliant Blue R-250 | 1 g |
| Methanol | ~500 mL |
| Acetic acid | ~150 mL |
| Molecular-weight marker | 1 vial |
| Water-saturated butanol | ~5 mL |
| Parafilm or 5 % agarose | for sealing the cassette |

Zymogram (optional): sodium acetate pH 5.0, Triton X-100, I₂ in KCl.

#### Samples and chromatography media

| Item | Amount |
| --- | --- |
| Sweet potato tuber | 1 tuber, ≥ 100 g (use 20.0 g) |
| Sephadex G-100 (default) | 4–5.3 g dry → 80 mL settled bed. G-150 and G-200 are discontinued — see S7.2 |
| Dialysis tubing 6–12 kDa MWCO | ~30 cm |
| Cheesecloth | 3–4 layers |

#### Consumables

| Item | Quantity |
| --- | --- |
| Centrifuge tubes 15/50 mL | ~20 |
| Microcentrifuge tubes | ~40 |
| Fraction collection tubes | 18, numbered, marked at 4.0 mL |
| Reaction tubes 10–15 mL | ~60 |
| Pipette tips 20/200/1000 µL | 2 boxes each |
| Dialysis clamps | 2 |
| Nitrile gloves | 1 box |
| Goggles | 1 pair |
| Chemical-waste containers (aqueous + organic) | 2 |
| Syringe (5-10 mL, no needle) | 1-2 |

#### Equipment

| Item | Specification |
| --- | --- |
| Mortar and pestle | pre-chillable, ≥ 100 mL |
| Beakers | 50, 100, 250, 500, 1000 mL |
| Graduated cylinders | 10, 50, 100, 500 mL |
| Volumetric flasks | 100 mL, 1000 mL |
| Pipettes | 100 µL, 200 µL, 1 mL, 5 mL — calibrated |
| Burette 50 mL + stand | for staged salt addition |
| Magnetic stirrer + bars | 1–2 sets |
| pH meter + probe |  |
| Balance | readable to 0.0001 g |
| Conductivity probe | optional |
| Refrigerator 4 °C / freezer −20 °C |  |
| Centrifuge | 4000 × g rotor, refrigerated |
| Chromatography column | ~80 mL bed, stopcock + frit |
| Clamps, retort stand, spirit level | for a vertical column |
| Vacuum desiccator + pump | degassing |
| Spectrophotometer | 540 nm and 280 nm and 260 nm |
| Water baths | 37 °C, 100 °C, 90 °C |
| Electrophoresis tank + power supply | 1.0 mm plates, 10-well comb |
| Staining tray, rocker, imager |  |

## Procedure

### P1 — Preparation

#### 1× PBS, pH 7.4 — 4 L

Supplementary: S1.1 mass formula · S1.2 target and masses · S1.3 hydrate check · S1.4 pH titration

| Ingredient | 4 L |
| --- | --- |
| NaCl | 32.02512 g |
| KCl | 0.80514 g |
| Na₂HPO₄ anhydrous | 5.6784 g |
| KH₂PO₄ | 0.979848 g |
| HCl, 1 M (pH adjust) | ≈ 11.3362836 mL |

- Weigh each salt to the masses above.

- Add the Na₂HPO₄ first and stir until clear before adding KH₂PO₄ then the rest in 3200 mL distilled water with stirring.

- Use a calibrated pH meter to adjust to pH 7.4 with 1 M HCl or NaOH, dropwise, probe immersed.

- Bring to 4000.0 mL using distilled water. Mix and re-check pH after dilution.

- Filter if cloudy. Store at 4 °C.

#### Assay buffer, pH 6.8 — 500 mL

Supplementary: S1.5 assay buffer masses · S1.6 CaCl₂ · S1.7 water budget

50 mM sodium phosphate.

| Ingredient | 500 mL |
| --- | --- |
| Na₂HPO₄·12H₂O | 8.9535 g |
| KH₂PO₄ | 3.40225 g |
| CaCl₂ (optional) | 1 mM = 0.05549 g |

- Dissolve both salts in 400 mL distilled water.

- Adjust to pH 6.8.

- Bring to 500.0 mL. Store at 4 °C.

#### Maltose standard, 1 mg/mL — 100 mL

Supplementary: S4.1 maltose molar mass in step 4

100.0 mg maltose, assay buffer to 100.0 mL.

- Weigh 100.0 mg maltose and bring to 100.0 mL with assay buffer.

- Mark the bottle. Store at 4 °C. Discard if it grows mould.

#### DNS reagent — 100 mL

Supplementary: S3.1 reference regression · S3.3 standards must match samples · S3.4 screening curve basis

| Ingredient | Amount | Final in 100 mL |
| --- | --- | --- |
| 3,5-Dinitrosalicylic acid | 1 g | 10 mg/mL |
| Rochelle salt (NaK tartrate) | 30 g | 300 mg/mL |
| NaOH, 2 M | 20 mL | 0.4 M delivered |
| Distilled water | to 100 mL | — |

Also make 100 mL of 2 M NaOH stock from 8 g NaOH.

- Dissolve 1 g DNS in ~20 mL distilled water with stirring. Warm to 40 °C if needed.

- Add 20 mL of 2 M NaOH — the solution turns deep red-brown.

- Add 30 g Rochelle salt in portions, stirring until dissolved.

- Bring to 100.0 mL. Mix and filter out undissolved particles.

- Store in an amber bottle at room temperature, protected from light. Discard if it precipitates or goes green or black.

#### Running buffer, 1× — 1 L

25 mM Tris, 192 mM glycine, 0.1 % SDS.

| Ingredient | Amount |
| --- | --- |
| Tris-HCl 1.5 M pH 8.8 | 16.7 mL |
| Glycine | 14.4 g |
| SDS | 1.0 g |

Dissolve in ~800 mL distilled water, bring to 1000 mL. Do not adjust pH. Use cold; keep at 4 °C and reuse once.

#### Laemmli buffer, 4× — 10 mL

| Ingredient | Amount |
| --- | --- |
| Tris-HCl 0.5 M pH 6.8 | 5 mL |
| SDS | 0.4 g |
| Glycerol | 2 mL |
| Bromophenol blue | ~4 mg |
| β-Mercaptoethanol | 0.4 mL — add fresh, in the hood |

Mix until dissolved. Use one part 4× to four parts sample.

#### Saturated (NH₄)₂SO₄ — 50 mL

Supplementary: S2.3 preparing the saturated stock · S2.1 saturation equation

- Put 50 mL distilled water in a beaker with the stirrer running.

- Add (NH₄)₂SO₄ a few grains at a time, stirring after each addition until the water is clear again.

- Keep adding until the salt stops dissolving and dry crystals are left in the beaker. That residue is what makes the solution saturated — not reaching 35.5 g.

- Expect to land within a few grams either side of 35.5 g. Anything over will simply sit undissolved.

- Put the beaker in the 4 °C refrigerator overnight. Do not filter out the crystals — they dissolve back in as the solution is used on ice.

#### Sephadex resin — dry, for an 80 mL bed

Supplementary: S7.2 why resin grade matters · S7 item 7 void volume

- Store the resin dry at 4 °C until Week 1. Weigh out 4–5.3 g of G-100 — Cytiva gives 15–20 mL bed volume per gram dry, so 80 mL of bed needs 80/20 to 80/15. Check the figure on your own bottle. G-150 and G-200 are discontinued; if the instructor wants a wide-range medium, expect Sephacryl S-400 rather than a Sephadex grade.

#### Also chill

Mortar, pestle, and a 60 mL aliquot of PBS.

- Put the mortar, pestle and the 60 mL aliquot of PBS in the refrigerator overnight.

#### Final product

| Product | Volume | Storage | Recorded |
| --- | --- | --- | --- |
| 1× PBS, pH 7.4 | 4 L | 4 °C |  |
| Assay buffer, pH 6.8 | 500 mL | 4 °C |  |
| Maltose standard, 1 mg/mL | 100 mL | 4 °C |  |
| DNS reagent | 100 mL | amber, room temp, dark |  |
| Running buffer, 1× | 1 L | 4 °C |  |
| Laemmli buffer, 4× | 10 mL | 4 °C, β-ME added fresh |  |
| Saturated (NH₄)₂SO₄ | 50 mL | 4 °C, in an ice bath when in use |  |
| Sephadex resin, dry | 4–5.3 g (G-100) | 4 °C |  |
| Mortar, pestle, 60 mL PBS aliquot | — | 4 °C, overnight |  |

### Week 1 — Extraction, precipitation, dialysis setup

#### Materials

Supplementary: S2.1 saturation equation · S2.2 why 47 mL is wrong · S2.4 staged addition arithmetic · S7.1 yield looks too low

| Item | Amount | From |
| --- | --- | --- |
| Sephadex, dry resin | 4–5.3 g (G-100) | P1 |
| Sweet potato tuber | 20.0 g | — |
| 1× PBS, cold | 60 mL extraction + 600 mL dialysis (200 mL × 3, 1:200) | P1 |
| Saturated (NH₄)₂SO₄ | 46.66667 mL | P1 |
| Dialysis tubing 6–12 kDa MWCO | ~30 cm, soaked and leak-tested | — |
| Cheesecloth | 3–4 layers | — |
| Burette 50 mL + stand |  | — |
| Centrifuge tubes 15/50 mL | ~10 | — |
| Mortar and pestle | chilled overnight | — |

| Equipment | Specification | Ready |
| --- | --- | --- |
| Centrifuge | 4000 × g, refrigerated, 4 °C |  |
| Water bath | 90 °C, for resin |  |
| Refrigerator | 4 °C |  |
| Ice | ~2 bags |  |

#### Procedure

Resin

- Suspend the weighed resin in ~105 mL PBS in a glass beaker. Put the beaker in a 90 °C water bath — G-100 needs 5 h at that temperature (72 h at room temperature). Start it first, then work on the extraction while it runs. Cover and hold at 4 °C once it is done.

Extraction

- Put the chilled mortar and pestle on an ice bath.

- Wash the tuber under running water, peel, discard any discoloured or soft tissue, and cut into ~5 mm cubes. Weigh exactly 20.0 g into a tared beaker. Keep on ice.

- Pour 30 mL cold PBS into the chilled mortar, then add the tissue.

- Grind in short firm strokes until no fragments remain and the paste is smooth. Work in 2–3 min bouts, returning the mortar to the ice bath between bouts.

- Filter through 3–4 layers of cheesecloth into a clean beaker, squeezing gently.

- Rinse the mortar, pestle and the residue on the cloth with the remaining 30 mL cold PBS.

- Bring the combined filtrate to 60.0 mL with cold PBS. This is Extract 1.

- Split: 20.0 mL → Extract 2, used immediately below. The remaining ≈ 40 mL into two aliquots at −20 °C as spare.

Precipitation

- Place the 20.0 mL Extract 2 in a beaker in an ice bath with the stirrer running.

- Add saturated (NH₄)₂SO₄ slowly from the burette, stirring continuously:

| Addition | This (mL) | Cumulative (mL) | Total with extract (mL) |
| --- | --- | --- | --- |
| 1 | 10.0 | 10.0 | 30.0 |
| 2 | 10.0 | 20.0 | 40.0 |
| 3 | 10.0 | 30.0 | 50.0 |
| 4 | 10.0 | 40.0 | 60.0 |
| 5 | 6.66667 | 46.66667 | 66.66667 |

- Continue stirring 20–30 min on ice after the last addition. Protein precipitates as white-to-cream floccules.

- Centrifuge 4000 × g, 10 min, 4 °C. Transfer the supernatant to a clean chilled tube, discard the pellet, and repeat once.

- Redissolve the pellet in 1.0 mL cold PBS until no fragments remain. This is Crude Enzyme 2. Record the actual volume used.

Dialysis

- Soak the tubing in distilled water for 20 min, then rinse. Leak-test a 5 mL piece for 5 min.

- Load the Crude Enzyme 2 in, clamp both ends, rinse the outside.

- Submerge the bag completely in 200 mL cold 1× PBS in a beaker in an ice bath — a 1:200 ratio against the 1.0 mL of sample. Stir gently and continuously.

- Leave 6 h. Change the external buffer at 2 h and 4 h — discard, add 200 mL fresh cold PBS each time. Harvest at 6 h; there is no third change.

#### Final product

| Product | Amount | State | Recorded |
| --- | --- | --- | --- |
| Crude Enzyme 2 | 1.0 mL | sealed in dialysis tubing, submerged in 200 mL PBS, 4 °C, 6 h |  |
| Spare Extract 1 aliquots | 2 × ~20 mL | −20 °C |  |
| Hydrated Sephadex slurry | ~5 g in ~105 mL PBS | 4 °C |  |

### Week 2 — Dialysis harvest, gel filtration

#### Materials

Supplementary: S7.2 resin grade and void volume · S8 why the column rules are what they are

| Item | Amount | From |
| --- | --- | --- |
| Crude Enzyme 2, desalted | 1.0 mL from the dialysis bag | Week 1 |
| Sephadex, hydrated | ~5 g in ~105 mL PBS | hydrated Week 1 |
| 1× PBS | 150 mL slurry + 10 mL + 160 mL wash + 160–240 mL equilibration + elution | P1 |
| Fraction collection tubes | 18, numbered, marked at 4.0 mL | — |
| Microcentrifuge tubes | ~40 | — |
| Conductivity probe | optional | — |

| Equipment | Specification | Ready |
| --- | --- | --- |
| Chromatography column | ~80 mL bed, stopcock + frit |  |
| Clamps, retort stand, spirit level | column must be vertical |  |
| Vacuum desiccator + pump | degassing |  |
| Centrifuge | 4000 × g, refrigerated, 4 °C |  |
| Spectrophotometer | 280 nm, for the effluent baseline |  |
| Refrigerator | 4 °C |  |

#### Procedure

Harvest

- Check the conductivity inside the bag against the external PBS — it should now match.

- Remove the bag, blot the outside dry, snip one end, transfer the contents to a chilled tube. Record the recovered volume.

- Clarify the Crude Enzyme 2 — 4000 × g, 30 min, 4 °C, twice, transferring the supernatant to a clean chilled tube after the first spin and discarding the pellet; keep the supernatant after the second. If it is still cloudy after the second spin, push it through a 0.45 µm low-protein-binding syringe filter instead. Record the clarified volume. This is Crude Enzyme 2, desalted.

- Keep on ice.

Resin

- Let the hydrated beads settle under gravity. Carefully decant the milky supernatant.

- Resuspend the settled bed in fresh PBS to a 50–75 % (v/v) slurry.

- Degas under vacuum 15–20 min.

Pack and equilibrate

- Mount the column vertically — check with the spirit level — and clamp firmly. Fit the bottom outlet and stopcock; close the stopcock.

- Pour 10 mL of degassed PBS into the bottom to wet the frit.

- Pour the slurry down the inside wall in one continuous stream, guiding with a glass rod.

- Open the outlet and drain by gravity until the bed height stops changing — 80 mL settled bed. Record the height. Never let the bed dry.

- Run 160 mL PBS through and discard that effluent.

- Equilibrate with 160–240 mL degassed PBS until effluent pH and A₂₈₀ are stable. Record both.

Load

- Drain the buffer until the meniscus is flush with the bed, then close the stopcock immediately.

- Apply 1.0–1.6 mL onto the centre of the bed in one smooth motion, without touching the bed surface.

- Leave it to enter with the outlet closed for 1 min, then open it.

- Rinse the column wall with 0.5 mL PBS and let it soak into the bed.

Elute

- Top up with PBS and open the outlet. Maintain 1–2 mL/min. Record the flow rate.

- Discard the void — the first 30 mL on G-100 (26–28 mL on G-50/G-75).

- Collect 4.0 mL into each of 12 numbered tubes.

- If A₂₈₀ in tube 12 is still above the buffer baseline, keep collecting in 4.0 mL fractions out to ~102 mL (tube 18).

- Keep the fractions on ice.

Store

- Split each fraction into two labelled aliquots — one for A₂₈₀ and DNS, one spare.

- Freeze both at −20 °C if analysing later. Thaw on ice, never at room temperature.

#### Final product

| Product | Amount | State | Recorded |
| --- | --- | --- | --- |
| Crude Enzyme 2, desalted | recorded volume | on ice |  |
| Gel filtration fractions | 12–18 × 4.0 mL | −20 °C, numbered |  |

Record: resin grade, settled bed volume and height, equilibration volume, effluent pH and A₂₈₀, void discarded, flow rate, clarified load volume, load activity, fractions collected.

### Week 3 — Screening and electrophoresis

#### Materials

Supplementary: S3.4 screening curve basis · S4.3 why the blanks are subtracted · S8 reading rules

| Item | Amount | From |
| --- | --- | --- |
| Gel filtration fractions, frozen | 12–18 tubes, thawed on ice | Week 2 |
| Extract 1, spare aliquot | ~5 mL | Week 1 |
| Crude Enzyme 2, spare aliquot | ~1 mL | Week 2 |
| 1 % soluble starch | 1 g in 100 mL assay buffer — batch A, fresh | P1 |
| Assay buffer | ~150 mL | P1 |
| Maltose standard | ~5 mL | P1 |
| DNS reagent | ~20 mL | P1 |
| Electrophoresis reagents | for one gel — fresh APS and TEMED | — |
| Reaction tubes 10–15 mL | ~20 | — |

| Equipment | Specification | Ready |
| --- | --- | --- |
| Spectrophotometer | 540 nm and 280 nm |  |
| Water bath | 37 °C, for the reaction |  |
| Water bath | 100 °C, for boiling |  |
| Electrophoresis tank + power supply | 1.0 mm plates, 10-well comb |  |
| Staining tray + rocker |  |  |
| Imager |  |  |
| Freezer | −20 °C |  |
| Ice | ~2 bags |  |

#### Procedure

Start the gel first, then screen while it runs.

- Make starch batch A: weigh 1 g starch, disperse it in a few mL of assay buffer, then add ~80 mL buffer. Heat to 80–90 °C for 30–60 s, swirling, until clear. Cool and bring to 100.0 mL. Use the same day.

- Thaw the fraction aliquots on ice.

- Read A₂₈₀ for every fraction, blanked against the same PBS used to equilibrate the column. Record.

- Assemble the plate sandwich with 1.0 mm spacers and comb; seal the bottom edge; fill with water and check for leaks, then drain.

- Cast the 12 % resolving gel to within 1 cm of the top. Overlay with water-saturated butanol. Polymerise 30–45 min.

- Pour off the overlay, rinse twice with water, then cast the 4 % stacking gel. Insert the comb, wells 1 mm deep. Set 20–30 min.

- Pre-run in 1× running buffer at 100 V for 15 min.

- Mix 4 volumes of sample with 1 volume of 4× Laemmli buffer — 10–20 µg per lane. Heat at 95 °C for 5 min, then spin briefly, in the hood. Expect a major band at ~56 kDa — the β-amylase subunit (Cheong et al. 1995, S12).

- Load and run: 70 V until the dye front clears the stacking gel (20–30 min), then 120 V until the front is ~1 cm from the bottom (60–90 min).

- Power down, open the cassette, cut off the stacking gel. Fix in 40 % methanol / 10 % acetic acid, 1 h, then stain in Coomassie R-250 for 1 h with rocking. Start destaining when you are ready for it.

| Lane | Content | Lane | Content |
| --- | --- | --- | --- |
| 1 | Molecular-weight marker | 6 | Crude Enzyme 2, 20 µg |
| 2 | Extract 1, 20 µg | 7 | Fraction, highest A₂₈₀ |
| 3 | Extract 1, 10 µg | 8 | Fraction, next |
| 4 | (blank / buffer) | 9 | Fraction, last collected |
| 5 | Extract 1, 5 µg | 10 | Peak fraction |

Screen the fractions while the gel runs.

- Build the screening curve on a 0.6 mL pre-DNS basis — 0, 0.12, 0.24, 0.36, 0.48, 0.60 mL maltose stock, each topped to 0.60 mL with buffer, then 1.0 mL DNS. Boil and cool exactly as the samples, then add 2.0 mL water to every standard — the standards must end at the same 3.6 mL as the samples or the curve is wrong.

- Run the screening reaction: 0.5 mL starch → pre-equilibrate 5 min at 37 °C → add 0.1 mL fraction → incubate exactly 10 min → add 1.0 mL DNS → boil 5 min → cool on ice → add 2.0 mL water → read at 540 nm against S0.

- Stagger the fraction additions and time on the first tube.

- Plot A₂₈₀ and screening activity against cumulative elution volume. One symmetric peak is the pass criterion.

- Pool the fractions at or above 50 % of the maximum A₂₈₀. Record the pooled volume. This is the Gel pool.

Protein Quantification (A₂₈₀/A₂₆₀ Spectrophotometry)

1. Turn on the spectrophotometer and set it to read at 280 nm and 260 nm.

2. Blank the instrument with PBS buffer for both wavelengths.

3. Measure A₂₈₀ and A₂₆₀ for Extract 1, Crude Enzyme 2, and the Gel pool. Note: If OD >1.5, dilute the sample and apply the dilution factor later.

4. Calculate Protein concentration (mg/mL) = (1.55 × A₂₈₀) - (0.77 × A₂₆₀).

5. Calculate A₂₈₀/A₂₆₀ ratio. A ratio near 1.7-2.0 indicates pure protein.

6. Calculate Specific Activity (U/mg) = Volumetric Activity (U/mL) / Protein Concentration (mg/mL). Record values in the Purification summary.

#### Final product

| Product | Amount | State | Recorded |
| --- | --- | --- | --- |
| Elution profile | 12–18 points | plotted |  |
| Gel pool | recorded volume | on ice, or −20 °C |  |
| SDS-PAGE gel | 1 | fixed, stained, destaining |  |

### Week 4 — Quantitative DNS

#### Materials

Supplementary: S4.1 the six steps · S4.2 symbols · S4.4 error budget · S5 worked example · S6 totals do not close

| Item | Amount | From |
| --- | --- | --- |
| Extract 1 | spare aliquot | Week 1 |
| Crude Enzyme 2, desalted | spare aliquot | Week 2 |
| Gel pool, pooled peak fractions | from Week 3 | Week 3 |
| 1 % soluble starch | 1 g in 100 mL assay buffer — batch B, fresh | P1 |
| Assay buffer | ~120 mL | P1 |
| Maltose standard | ~5 mL | P1 |
| DNS reagent | ~40 mL | P1 |
| Reaction tubes 10–15 mL | ~40 | — |
| Stopwatch | for the 10 min incubations | — |

| Equipment | Specification | Ready |
| --- | --- | --- |
| Spectrophotometer | 540 nm |  |
| Water bath | 37 °C, for the reaction |  |
| Water bath | 100 °C, for boiling |  |
| Ice | ~2 bags |  |
| Stopwatch | for the 10 min incubations |  |

#### Procedure

- Make starch batch B — 1 g in 100 mL assay buffer, made the same way as batch A. Use the same day.

- Build the standard curve on a 1.0 mL pre-DNS basis — 0, 0.2, 0.4, 0.6, 0.8, 1.0 mL maltose stock, each topped to 1.0 mL with buffer.

- Add 1.0 mL DNS to every tube. Boil 5 min in a 100 °C bath, timing from when the tubes go in.

- Cool in ice water to room temperature. Add 8.0 mL water. Mix by inversion. Read at 540 nm, zeroed on S0.

- Fit A₅₄₀ = a x+b by least squares. Record a, b and R2. Accept R2≥0.98. If S5 reads above A = 2.0–2.5, reduce the top standard and rebuild. Never extrapolate beyond S5.

- One curve, all three stages. Do not build a second curve later.

Dilution screen — Extract 1 only

- Prepare four levels of Extract 1 — neat, 1:5, 1:10, 1:20 — 2.5 mL of each, kept on ice:

| Level | Extract (µL) | Buffer (µL) | Total (µL) | DF​ |
| --- | --- | --- | --- | --- |
| Neat | 2500 | 0 | 2500 | 1 |
| 1:5 | 500 | 2000 | 2500 | 5 |
| 1:10 | 250 | 2250 | 2500 | 10 |
| 1:20 | 125 | 2375 | 2500 | 20 |

2.5 mL covers five tubes of 0.5 mL — three samples, one enzyme blank and one substrate blank. The neat level is expected to read above S5 and is discarded; use it to confirm the assay saturates, not to calculate. 8. Make the 1:20 from the 1:5 stock — 500 µL of 1:5 + 2000 µL buffer. 9. For each level, run the reaction in steps 11–19 below with three sample tubes, one enzyme blank and one substrate blank. 10. Use the most dilute level whose corrected A₅₄₀ falls strictly inside the standard-curve range. If even 1:20 exceeds S5, extend to 1:40 or 1:80.

Quantitative reaction — repeat for Crude Enzyme 2 and the Gel pool at the selected level

| Tube | 1 % starch | Assay buffer | Diluted enzyme | DNS | When |
| --- | --- | --- | --- | --- | --- |
| Sample (× 3) | 0.5 mL | 0 | 0.5 mL | 1.0 mL | after incubation |
| Blank enzyme (BE) | 0.5 mL | 0 | 0.5 mL | 1.0 mL | before enzyme |
| Blank substrate (BS) | 0.5 mL | 0.5 mL | 0 | 1.0 mL | after incubation |

- Pipette 0.5 mL starch into every tube.

- Pre-equilibrate 5 min at 37 °C. Add 0.5 mL buffer to BS now.

- Add 1.0 mL DNS to BE only — before any enzyme goes in.

- Add 0.5 mL diluted enzyme to the 3 sample tubes and to BE. Stagger the additions and start the stopwatch on the first.

- Incubate exactly 10 min at 37 °C.

- Add 1.0 mL DNS to the 3 sample tubes and to BS.

- Boil 5 min, cool in ice water.

- Add 8.0 mL water, mix by inversion.

- Read at 540 nm against S0. Record every tube before taking the mean.

- Calculate activity for all three stages — chain in S4. Fill the purification summary.

- Destain and image the Week 3 gel.

Enzyme Immobilization (Ca-Alginate Beads)

1. Prepare a 2–3% (w/v) Sodium Alginate solution in distilled water. Mix thoroughly until completely dissolved.

2. Prepare a 0.2 M CaCl₂ solution (dissolve ~2.22 g CaCl₂ in 100 mL distilled water) and keep it cold (4°C).

3. Mix the purified β-amylase (Gel pool) with the Sodium Alginate solution in a 1:1 or 1:2 ratio. Stir gently to avoid bubbles.

4. Draw the mixture into a 5–10 mL syringe (without needle or with a large gauge needle).

5. Slowly add the mixture drop-by-drop into the cold 0.2 M CaCl₂ solution from a height of ~5 cm. The drops will instantly form spherical Ca-alginate beads.

6. Allow the beads to harden in the CaCl₂ solution for 30 minutes at 4°C.

7. Filter the beads and wash them twice with cold distilled water or PBS buffer to remove excess CaCl₂ and unbound enzyme.

8. The immobilized enzyme beads can now be stored at 4°C or tested for activity by incubating them with the starch solution.

#### Final product

| Product | State | Recorded |
| --- | --- | --- |
| Curve a / b / R2 | recorded |  |
| Extract 1 — U/mL, total U | recorded |  |
| Crude Enzyme 2 — U/mL, total U | recorded |  |
| Gel pool — U/mL, total U | recorded |  |
| Yield at each step | recorded |  |
| SDS-PAGE image | destained, band position recorded |  |

## Data sheets

### Gel filtration elution log

| Fraction | Cumulative (mL) | Colour | A₂₈₀ | A₅₄₀ screen | Screen U/mL | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |
| 18 |  |  |  |  |  | (continue while A₂₈₀ is above baseline) |

### DNS assay worksheet — one block per dilution level

|  | 1:5 | 1:10 | 1:20 | Other |
| --- | --- | --- | --- | --- |
| A₅₄₀ sample 1 |  |  |  |  |
| A₅₄₀ sample 2 |  |  |  |  |
| A₅₄₀ sample 3 |  |  |  |  |
| Asample = mean |  |  |  |  |
| ABE​ |  |  |  |  |
| ABS​ |  |  |  |  |
| Acorr​ |  |  |  |  |
| c (mg/mL) |  |  |  |  |
| m (mg) |  |  |  |  |
| n (µmol) |  |  |  |  |
| U/mL |  |  |  |  |
| Inside the curve? (Y/N) |  |  |  |  |

### Purification summary

|  | Extract 1 | Crude Enzyme 2 | Gel pool |
| --- | --- | --- | --- |
| Total volume (mL) |  |  |  |
| Dilution factor used |  |  |  |
| Asample (mean) |  |  |  |
| ABE​ |  |  |  |
| ABS​ |  |  |  |
| Acorr​ |  |  |  |
| c (mg/mL) |  |  |  |
| n (µmol) |  |  |  |
| Volumetric activity (U/mL) |  |  |  |
| Total activity (U) |  |  |  |
| Yield (%) |  |  |  |
| A₂₈₀ |  |  |  |
| A₂₈₀ / A₂₆₀ |  |  |  |
| Protein (mg/mL) |  |  |  |
| Specific Activity (U/mg) |  |  |  |
| Main band (kDa) |  |  |  |

## Volumes

| Step | Volume | Session |
| --- | --- | --- |
| Extraction buffer | 30 mL grind + 30 mL rinse = 60 mL for 20 g | 1 |
| Extract 2 | 20.0 mL | 1 |
| Saturated (NH₄)₂SO₄ to add | 46.66667 mL | 1 |
| Final precipitation volume | 66.66667 mL | 1 |
| Pellet redissolved in | 1.0 mL | 1 |
| Dialysis buffer | 1 L, changed at 2, 4, 6 h | 1–2 |
| Resin hydration | ~5 g in ~105 mL PBS | 1 |
| Bed volume Vc | 80 mL | 2 |
| Fines wash / equilibration | 160 mL / 160–240 mL | 2 |
| Column load | 1.0–1.6 mL (1.25–2.0 % of Vc) | 2 |
| Void V0 discarded | 26–28 mL (G-50/G-75) / 30 mL (G-100) | 2 |
| Fractions | 12 × 4.0 mL, continue to tube 18 if needed | 2 |
| DNS reaction | 0.5 + 0.5 mL pre-DNS; + 1.0 mL DNS; + 8.0 mL water = 10.0 mL | 4 |
| Screen reaction | 0.1 + 0.5 mL pre-DNS; + 1.0 mL DNS; + 2.0 mL water = 3.6 mL | 3 |

## Waste and cleanup

Dispose of all (NH₄)₂SO₄, DNS and Coomassie/destain solutions as aqueous chemical waste — none of it goes down a sink. The Coomassie stain is methanol- and acid-based and needs chemical collection. Gloves, gel casts and electrophoresis buffer are chemical waste, not domestic.